In the rt85 eprom RX channel 0 starts at byte  $0001

10 starts at byte  $0041

20    "    "   "   $0081

30                 $00C1

40                 $0101

50                 $0141

60                 $0181

In the rt85 eprom TX channel 0 starts at byte  $0401

10 starts at byte  $0441

20    "    "   "   $0481

30                 $04C1

40                 $0501

50                 $0541

60                 $0581

Each channel data consists of 3 bytes, and includes frequency, CTCSS, checksum.   Channels are spaced to start at 4 byte intervals, leaving a one byte gap between each channel. Some are used for the scanlist (see below)

**********************

SCANNING

Scanlist starts at  byte  $00,$04,$08,$0C

$10,$14,$18

The channel number to scan is stored in DECIMAL

$0001     1st scan channel

$0004     2nd scan channel

$0008     3rd  "      "

$000C     4th  "      "

$0010     5th  "      "

$0014     6th  "      "

$0018     7th  "      "

***********************

PERSONALITY

Personality bytes are at $03F0, $03F1, $03F2, $03F3

$03F0     maximum channel number  (decimal I think)

------------------------------------------------------

$03F1

digit 1

| CTCSS Decoder fitted? x | x | x | x |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | x |  |  | x | x |  |  |
| x |  | x |  | x |  | x |  |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Busy Lamp delay?

Selcall fitted?

Programming code    $

digit 2

| Status No. Required? x | x | x | x |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | x |  |  | x | x |  |  |
| x |  | x |  | x |  | x |  |
| E | C | A | 8 | 6 | 4 | 2 | 0 |
| F | D | B | 9 | 7 | 5 | 3 | 1 |

Inhibit TX when BUSY LED ON?

Inhibit TX when OPEN LED OFF?

CODE with silent switch active            CODE with silent switch inhib.

---------------------------------------------------------------------------------------------

$03F2

digit 1

| PTT rel (mSec) hold on delay 350 | 300 | 250 | 200 | 150 | 100 | 50 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Programming code

digit 2

| TX Timeout (Sec) 210 | 180 | 150 | 120 | 90 | 60 | 30 | no |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Programming code

---------------------------------------------------------------------------------------------------------

$03F3

Scan stops with  ctcss/carrier	CTCSS TONE      I      CARRIER

| Scan HOLD ON Delay 1.3 | 2.5 | 5.0 | 7.5 | 1.3 | 2.5 | 5.0 | 7.5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 51 | 61 | 71 | 42 | 52 | 62 | 72 |
| 01 | 11 | 21 | 31 | 02 | 12 | 22 | 32 |

CODE with scan 0.2sec/ch            CODE with scan 0.4sec/ch

Auxiliary Code

0	No ctcss		10	173.8	A	20	123.0	A

1	241.8	B	11	167.9	B	21	118.8	B

2	233.6	A	12	162.2	A	22	114.8	A

3	225.7	B	13	156.7	B	23	110.9	B

4	218.1	A	14	151.4	A	24	107.2	A

5	210.7	B	15	146.2	B	25	103.5	B

6	203.5	A	16	141.3	A	26	100.0	A

7	192.8	B	17	136.5	B	27	 94.8	B

8	186.2	A	18	131.8	A	28	 88.5	A

9	179.9	B	19	127.3	B	29	 82.5	B

30	 77.0	A

31	 71.9	B